TCP Congestion Control – Complete Lab Task

MOHAMMED NUMAAN AHMED

2503B05127

Q1. LAB TASK: TCP Congestion Control Simulation

Question:
Using a TCP congestion control simulator (Python or any tool of your choice), simulate and analyze TCP Reno under three different packet-loss conditions:

- 0.1% loss

- 1% loss

- 2% loss

For each case:

- Plot the Congestion Window (cwnd) vs Time for at least 20 seconds of simulation.

- Compute the average throughput (in MSS/second or Mbps).

- Explain how packet loss affects the behavior of TCP Reno, specifically discussing:

  - Slow Start

  - Congestion Avoidance

  - Multiplicative Decrease (cwnd halving)

- Compare the three cases and write a short conclusion on how increasing loss impacts network performance.

Reference:

# tcp_sim.py

# Simple discrete-time TCP congestion control simulator (Reno-like)

# Usage: python tcp_sim.py

import numpy as np

import matplotlib.pyplot as plt

import random

# --- Simulation parameters ---

RTT = 0.05               # seconds (50 ms)

sim_time = 20.0          # seconds

mss = 1.0                # normalized MSS (units)

loss_prob = 0.01         # per-packet loss probability (Bernoulli)

packets_per_seg = 1      # one packet per MSS in this model

# TCP behavior params

algorithm = "reno"       # "reno", "tahoe"

initial_cwnd = 1.0       # in MSS

ssthresh_init = 64.0     # in MSS

rto = 1.0                # simplified timeout (s)

# --- State variables ---

t = 0.0

cwnd = initial_cwnd

ssthresh = ssthresh_init

time_steps = []

cwnd_trace = []

sent_packets_total = 0

acked_packets_total = 0

# For throughput measurement

bytes_acked = 0.0

# Helper: simulate sending cwnd-sized window; return True if loss detected

def send_window(cwnd):

# we model loss per packet with independent Bernoulli trial

segs = int(max(1, round(cwnd / mss)))

lost = False

acks = 0

for _ in range(segs):

sent_packets_total_local = 1

if random.random() < loss_prob:

lost = True

else:

acks += 1

return lost, acks

# Main loop: step in RTT-sized intervals

while t < sim_time:

# record

time_steps.append(t)

cwnd_trace.append(cwnd)

# determine phase: slow start or congestion avoidance

in_slow_start = (cwnd < ssthresh)

# send window, get ack/loss (this is simplification; real TCP tracks per-packet ACKs)

lost, acks = send_window(cwnd)

# update bytes acked

bytes_acked += acks * mss

# Simple Reno behavior

if lost:

# If Tahoe: go to slow start with cwnd = 1

if algorithm == "tahoe":

ssthresh = max(cwnd / 2.0, 2.0)

cwnd = 1.0

elif algorithm == "reno":

# Reno: multiplicative decrease, enter congestion avoidance

ssthresh = max(cwnd / 2.0, 2.0)

cwnd = max(cwnd / 2.0, 1.0)

# assume loss consumed this RTT

else:

# no loss -> increase cwnd

if in_slow_start:

cwnd = cwnd * 2.0  # exponential growth (per-RTT doubling)

else:

cwnd = cwnd + 1.0  # 1 MSS per RTT (additive increase)

# Cap cwnd to a reasonable value to avoid runaway in this simple sim

cwnd = min(cwnd, 1000.0)

# advance time by one RTT

t += RTT

# Convert to throughputs: bytes_acked / sim_time (in MSS units)

throughput = bytes_acked / sim_time

# Plot cwnd trace

plt.figure(figsize=(10,4))

plt.step(time_steps, cwnd_trace, where='post')

plt.xlabel("Time (s)")

plt.ylabel("cwnd (MSS)")

plt.title(f"TCP {algorithm} simulation: loss={loss_prob}, RTT={RTT}s, throughput={throughput:.2f} MSS/s")

plt.grid(True)

plt.show()

print(f"Sim done. Throughput (MSS/s): {throughput:.3f}")

CODE:

# -*- coding: utf-8 -*-

"""Welcome To Colab

Automatically generated by Colab.

Original file is located at

https://colab.research.google.com/notebooks/intro.ipynb

"""

!pip install python-docx --quiet

import random

import math

import numpy as np

import matplotlib.pyplot as plt

from pathlib import Path

import shutil

from docx import Document

from docx.shared import Inches

RTT = 0.05

SIM_TIME = 20.0

MSS_BYTES = 1460

LOSS_VALUES = [0.001, 0.01, 0.02]

INITIAL_CWND = 1.0

SSTHRESH_INIT = 64.0

RANDOM_SEED_BASE = 42

MAX_CWND = 1000.0

def simulate_tcp_reno(loss_prob, rtt=RTT, sim_time=SIM_TIME, seed_offset=0):

"""Simulate a simple TCP Reno on RTT-sized steps.

Returns: (time_steps_array, cwnd_array, throughput_mss_per_s, throughput_mbps)"""

random.seed(RANDOM_SEED_BASE + seed_offset)

t = 0.0

cwnd = float(INITIAL_CWND)

ssthresh = float(SSTHRESH_INIT)

time_steps = []

cwnd_trace = []

bytes_acked = 0.0

while t < sim_time - 1e-9:

time_steps.append(t)

cwnd_trace.append(cwnd)

in_slow_start = (cwnd < ssthresh)

segs = max(1, int(math.floor(cwnd + 1e-9)))

lost = False

acks = 0

for _ in range(segs):

if random.random() < loss_prob:

lost = True

else:

acks += 1

bytes_acked += acks * MSS_BYTES

if lost:

ssthresh = max(cwnd / 2.0, 2.0)

cwnd = max(cwnd / 2.0, 1.0)

else:

if in_slow_start:

cwnd = cwnd * 2.0

else:

cwnd = cwnd + 1.0

cwnd = min(cwnd, MAX_CWND)

t += rtt

throughput_mss_per_s = (bytes_acked / MSS_BYTES) / sim_time

throughput_mbps = (bytes_acked * 8.0) / (sim_time * 1e6)

return np.array(time_steps), np.array(cwnd_trace), throughput_mss_per_s, throughput_mbps

output_dir = Path("tcp_sim_output")

output_dir.mkdir(exist_ok=True)

results = []

for i, loss in enumerate(LOSS_VALUES):

ts, cwnd, thr_mss_s, thr_mbps = simulate_tcp_reno(loss, seed_offset=i)

results.append((loss, ts, cwnd, thr_mss_s, thr_mbps))

plt.figure(figsize=(10,4))

plt.step(ts, cwnd, where='post')

plt.xlabel("Time (s)")

plt.ylabel("cwnd (MSS)")

pct = {0.001: "0.1%", 0.01: "1.0%", 0.02: "2.0%"}.get(loss, f"{loss*100:.3f}%")

plt.title(f"TCP Reno: loss={pct}  (RTT={RTT}s, sim={SIM_TIME}s)")

plt.grid(True)

plt.tight_layout()

fname = output_dir / (f"cwnd_{int(loss*10000)}_{int(loss*1000)}pct.png")

if loss == 0.001:

fname = output_dir / "cwnd_0.1pct.png"

elif loss == 0.01:

fname = output_dir / "cwnd_1pct.png"

elif loss == 0.02:

fname = output_dir / "cwnd_2pct.png"

plt.savefig(fname, dpi=150)

plt.show()

plt.close()

plt.figure(figsize=(11,5))

for loss, ts, cwnd, thr_mss_s, thr_mbps in results:

label = f"{(loss*100):.2f}% loss"

plt.step(ts, cwnd, where='post', label=label)

plt.xlabel("Time (s)")

plt.ylabel("cwnd (MSS)")

plt.title("TCP Reno cwnd vs Time (comparison)")

plt.legend()

plt.grid(True)

plt.tight_layout()

combined_fname = output_dir / "cwnd_comparison.png"

plt.savefig(combined_fname, dpi=150)

plt.show()

plt.close()

print("Simulation summary:")

for loss, ts, cwnd, thr_mss_s, thr_mbps in results:

pct = {0.001: "0.1%", 0.01: "1.0%", 0.02: "2.0%"}.get(loss, f"{loss*100:.3f}%")

print(f"- Loss = {pct:<6} | Avg throughput = {thr_mss_s:.3f} MSS/s  |  {thr_mbps:.3f} Mbps")

report_doc = Document()

report_doc.add_heading("TCP Reno Simulation Report", level=1)

report_doc.add_paragraph(f"Simulated for RTT={RTT}s, simulation time={SIM_TIME}s, MSS={MSS_BYTES} bytes.")

report_doc.add_paragraph("Loss scenarios: 0.1%, 1%, 2%")

for loss, ts, cwnd, thr_mss_s, thr_mbps in results:

pct = {0.001: "0.1%", 0.01: "1.0%", 0.02: "2.0%"}.get(loss, f"{loss*100:.3f}%")

report_doc.add_heading(f"Loss = {pct}", level=2)

report_doc.add_paragraph(f"Average throughput: {thr_mss_s:.3f} MSS/s  ({thr_mbps:.3f} Mbps)")

obs = ("Observation: With higher loss the congestion window experiences "

"more frequent multiplicative decreases (cwnd halving), reducing average cwnd and throughput.")

report_doc.add_paragraph(obs)

img_name = None

if loss == 0.001:

img_name = output_dir / "cwnd_0.1pct.png"

elif loss == 0.01:

img_name = output_dir / "cwnd_1pct.png"

elif loss == 0.02:

img_name = output_dir / "cwnd_2pct.png"

if img_name and img_name.exists():

report_doc.add_picture(str(img_name), width=Inches(6))

report_doc.add_heading("Comparison: cwnd vs Time", level=2)

if combined_fname.exists():

report_doc.add_picture(str(combined_fname), width=Inches(6))

report_doc.add_heading("Conclusion", level=2)

report_doc.add_paragraph(

)

report_path = output_dir / "TCP_Reno_Report.docx"

report_doc.save(report_path)

print(f"\nSaved report to: {report_path}")

uploaded_path = Path("/mnt/data/Assignment-6.docx")

if uploaded_path.exists():

shutil.copy(uploaded_path, output_dir / uploaded_path.name)

print(f"Copied uploaded file to: {output_dir / uploaded_path.name}")

else:

print(f"Uploaded path not found at {uploaded_path} (this path was included per your session history).")

OUTPUT:

Simulation summary:

- Loss = 0.1%   | Avg throughput = 687.200 MSS/s  |  8.026 Mbps

- Loss = 1.0%   | Avg throughput = 279.550 MSS/s  |  3.265 Mbps

- Loss = 2.0%   | Avg throughput = 156.800 MSS/s  |  1.831 Mbps